The DECOMPXR software described in this User's Guide is furnished under a license agreement.  The software may be used or backed up only in accordance with the terms of this agreement.  No part of this User's Guide may be reproduced or transmitted in any form or by any means, electronic or mechanical, including photocopying, without the prior written permission of Etudes Recherches Matériaux.

Information within this User's Guide is subject to change without notice and does not represent product specification or commitment on the part of Etudes Recherches Matériaux.

TABLE OF CONTENTS

Table of contents	3

Introduction	5

Requirements	7

File list	9

Installation procedure	11

Installation procedure	12

Configuration procedure	13

Protection procedure	14

Reference guide	16

File	16

Load	16

Directory	17

Edit	17

Parameters	17

Range	17

Elem. Curves	18

Iterations	18

Output	18

K2	18

Pre-Treat	19

Radiation	19

Simulation	19

Batch Calc.	20

Save Results	20

Go	20

Routine procedure	20

Parameters+Batch Calc	25

Quit	26

Troubleshooting - Error messages	27

Auxiliary programs	31

Command file generation	31

LIST.EXE	31

Format translator programs	32

Program RAW1DAT.EXE	32

Program RAW2DAT.EXE	32

Program UXDDAT.EXE	32

Program APDDAT.EXE	33

Program SMSDAT.EXE	33

Program TRUDAT.EXE	33

Examples	34

Introduction	34

Example 1	34

Remark on natural peak shape	35

Example 2	35

Example 3	38

Conclusion	40

References	42

Annex 1 - File formats	44

Format of the configuration file DECOMP.CFG	44

Format of a data file	44

Format of a result file	45

Format of the command files used by DECOMPXR	46

Format of the command files used by translation programs	46

Index	47

Addition to the user's guide	49

INTRODUCTION

Polyphasic samples are very common in natural low grade systems because of the major influence of kinetics on numerous mineral transformations, especially clay mineral transformations (Velde, 1985).  A numerical decomposition treatment of X-ray diffraction patterns makes the identification of the different phases present in such samples much easier, even if these phases have closely related crystallographic characteristics:

- In late stage diagenesis, Lanson (1990), Lanson and Champion (1991), Lanson and Besson (1992), and Lanson and Velde (1992) evidenced ordered illite/smectite mixed-layered coexisting with discrete illite and detrital (?) mica.

- In soils, Righi and Meunier (1991) showed the simultaneous presence of mica, chlorite, vermiculite, interstratified mica/vermiculite and possibly interstratified chlorite/vermiculite.

- In experimental hydrothermal alteration of sedimentary clay samples for nuclear waste disposal simulation, Bouchet et al. (1992) evidenced the coexistence of random mixed-layer kaolinite/smectite and discrete smectite.

- In hydrothermal samples, randmoly interstratified mixed-layer chlorite/corrensite and discrete chlorite were found (Beaufort et al., 1997).

Without a decomposition treatment, the identification of these phases is very difficult because most of the existing identification procedures are not conceived to separate the contributions of the various clay phases.

The coexistence of various illitic phases with closely related but distinct crystallographic characteristics, which seems to be a very common occurrence in natural diagenetic systems and metabentonites, has been evidenced not only with X-ray diffraction (XRD) but also with transmission electron microscope (TEM) observations (Lanson and Champion, 1991; Freed and Peacor, 1992) as well as with chemical analyses (energy dispersive spectrometry) performed with a TEM on individual particles (Lanson and Champion, 1991).  When such a coexistence is observed, a great care should be taken in using identification procedures which are not conceived to separate the contributions of the various illitic phases, and are likely to give you incorrect results.

DECOMPXR is a peak decomposition program, which will provide you with accurate information concerning position, width, and intensity of the peaks associated with all the different phases present in a sample.  This information is especially useful when these peaks are overlapping, and thus when it is impossible to obtain the necessary information with routine XRD data treatment.  However, it does not give you information on the nature of these phases.  Phase identification is made much easier by peak decomposition but remains a mandatory step.

The algorithms used in this program, its limitations, its performances, and its reproducibility are described by Lanson (1990), Lanson (1992), Lanson and Champion (1991), Lanson and Besson (1992), and Lanson and Velde (1992).

REQUIREMENTS

Computer: IBM AT or 100% compatible equipped with a processor Intel 80286 or more recent, and one hard disk with a minimum of one Megabyte of free disk space.  Because most of the time you spend using DECOMPXR is calculation time, it is much more agreeable to run it on a fast system (e.g. processor 80386 25/33 MHz, with arithmetic coprocessor).

Operating system: MS-DOS version 3.2 or later, or 100% compatible operating system.

Memory: 640 Kb of RAM are required to run this program; on smaller systems problems with graphic initialization can occur after calculation (especially with high resolution VGA color screens).

Graphic card: High definition EGA or VGA graphic card and color screen required.

Mouse: Compatible Microsoft (bus or serial) required, some options do not run without a mouse.

Printer: Not required, but strongly recommended.

Arithmetic coprocessor: Not required, but strongly recommended, as most of the running time is calculation time.  If present, the arithmetic coprocessor is detected and used; if not, its presence is emulated by DECOMPXR.

Remark: This program requires numerous disk accesses to run.  To reduce the calculation time, and to save your hard disk, it is strongly recommended that you define a virtual drive in the RAM of your computer.  To install the virtual disk driver you must add the following command line to your CONFIG.SYS file and reboot the system:

DEVICE=Path\Util.SYS DiskSize SectorSize Entries /E.

Path\Util.SYS: Indicate the name (usually RAMDRIVE.SYS or VDISK.SYS) and the path of the virtual disk driver.  For example, if the driver RAMDRIVE.SYS is located in the directory \DOS\ on the disk C, you have to type C:\DOS\RAMDRIVE.SYS.

DiskSize is the virtual disk size in kbytes.  128 kbytes are necessary to run DECOMPXR.

SectorSize is the number of bytes per sector.  You can leave this entry blank, and use the default value (usually 512).

Entries: Number of directory entries.  You can leave this entry blank, and use the default value (usually 64 files).  The minimum value for DECOMPXR is 8.

/E is usually the extended memory switch, to create the virtual drive in the memory above 1 Mb (option strongly recommended).  To make it possible for your virtual drive driver to use the extended memory, the configuration of your system must provide this memory, and the command line for the extended memory driver (e.g. HIMEM.SYS) must be located before the one for the virtual disk driver in the CONFIG.SYS file.

See your operating system user's guide for additional information.  If Windows is installed on your computer, be aware of potential compatibility problems between your virtual disk driver and SMARTDRV.SYS, which presence is not required to run the graphic environment Windows.

Remark: After calculation, experimental and fitted patterns, as well as elementary curves, are displayed on a high resolution graphic (EGA or VGA) color screen.  To copy this screen on your printer, you need a hardcopy utility such as GRAPHICS.COM, PJSCREEN.EXE, among many others; if you are using a high definition VGA-type screen check that your utility allows you to copy such screens (most of the GRAPHICS.COM utilities provided with DOS 3.3 or earlier do not).  This utility is memory resident and must be loaded before running DECOMPXR.

FILE LIST

The DECOMPXR disk you received contains the following files:

INSTALL.EXE: Program to install DECOMPXR.

INSTDCP.EXE: Program to write the protection code in the configuration file (DECOMP.CFG) in case the installation procedure had been stopped before being completed.

DECOMPXR.EXE: X-ray patterns decomposition program.

LIST.EXE: Program to create command files either to translate various data file formats to the format used by DECOMPXR, or to use for batch calculation in DECOMPXR.

RAW1DAT.EXE:Program to translate automatically the raw data files created by the DIFFRAC-AT (up to version 3.0 excluded) diffraction management system to the format used by DECOMPXR.

RAW2DAT.EXE:Program to translate automatically the raw data files created by the DIFFRAC-AT (version 3.0 and later) diffraction management system to the format used by DECOMPXR.

UXDDAT.EXE:Program to translate automatically the ASCII data files using the "universal data file format" created by Socabim (DIFFRAC-AT package) to the format used by DECOMPXR.

SMSDAT.EXE:Program to translate automatically the ASCII data files created by the Siemens D5000 diffraction management system running on Vax, and transferred to an IBM AT-compatible computer to the format used by DECOMPXR.

TRUDAT.EXE:Program to translate automatically the ASCII data files created by the Newmod® program running on IBM AT-compatible computers (or on Macintosh computers after transfer to an IBM AT-compatible format) to the format used by DECOMPXR.

APDDAT.EXE:Program to translate automatically the raw data files created by the Philips APD diffraction management system running on IBM-compatible computers to the format used by DECOMPXR.

EXAMPLES\*.DAT: Example files.

These files are not copy protected, and may be copied, for backup purpose only, with any copy procedure.

INSTALLATION PROCEDURE

To install DECOMPXR on your hard disk:

1) Insert the DECOMPXR disk in your drive A:

2) Type A:INSTALL

3) Follow the step-by-step installation procedure described below.

The installation procedure is self-explanatory.  Consequently, the following section will essentially contains the messages displayed by the INSTALL.EXE program (in bold) with a few comments when necessary.  After each entry INSTALL usually displays a message with entry it actually took into account to make it possible for you to correct typing errors, or possible (?) incorrect entry format.

INSTALLATION FOR X-RAY PATTERNS DECOMPOSITION PROGRAM
DECOMPXR - MARCH 1997 version

This program will first copy the various DECOMPXR files from your drive A: to the specified disk and path.

Then, it will ask you for informations on your diffractometer to configure correctly the DECOMP.CFG file.

Finally, it will give you a code specific to your machine.  You will need to call E.R.M. to obtain the protection code to be included in the DECOMP.CFG file.

Press any key to install DECOMPXR - Ctrl+Break to stop installation

You can run the installation procedure as many times as you want.  However, each time it will copy the DECOMPXR files on your hard disk, so be sure to delete the unnecessary copies to save disk space.  You can also delete the unnecessary format translator programs if you want to minimize the space occupied on your hard disk.

If you hit <Ctrl>+<Break>, you will need to run again the installation procedure or copy all the DECOMPXR program files on your hard disk, and create the DECOMP.CFG file according to the format described in Annex 1.  In anycase, you will have to run the INSTDCP.EXE program to get the site-specific code and call E.R.M. to obtain your protection code.

SITE SPECIFIC CODE CALCULATION FOR X-RAY PATTERNS DECOMPOSITION PROGRAM
DECOMPXR - MARCH 1997 version

Site specific code calculated by the installation procedure: Site-specific code

To know the protection code to be included in the DECOMP.CFG file, please note this eight characters code and contact either:

Dr. Alain Bouchet	Dr. Bruno Lanson
E.R.M. - Batiment GON	Environmental Geochemistry Group
Faculté des Sciences	LGIT - IRIGM

40 Avenue Recteur Pineau	BP 53
86022 Poitiers cedex	38041 Grenoble cedex 9
FRANCE	FRANCE

Phone# +33 (0)5 49 46 18 11	Phone# +33 (0)4 76 82 80 17

Fax# +33 (0)5 49 45 40 26	Fax# +33 (0)4 76 82 81 01

E-mail : Bruno.Lanson@obs.ujf-grenoble.fr

To complete the installation at once, you will need the protection code specific to your computer.  This code is given by E.R.M. as a fuction of the site specific code displayed on the above screen.  However, you can run the installation procedure INSTALL.EXE up to the protection section and run INSTDCP.EXE later from the directory where the DECOMP.CFG configuration file is located.  This latter program will execute only the protection section of this installation procedure.  In any case, the protection code given by your E.R.M. correspondent must be written in the DECOMP.CFG to complete the installation.

Installation section.  This part of the installation procedure asks you the disk and directory on which you want to install the DECOMPXR program.  It will copy the various program files and example data files in the specified directory.

INSTALLATION PROCEDURE FOR X-RAY PATTERNS DECOMPOSITION PROGRAM
DECOMPXR - MARCH 1997 version

This procedure will copy the various DECOMPXR files from A: to the specified disk and path.  If the specified directory does not exist it will be created.

Enter the disk on which you want to install DECOMPXR (C:):

Disk on which DECOMPXR will be installed: Entry

Enter the name of the disk, or just press <Enter> for the default disk (C:).  INSTALL checks if the specified disk exists; if not, it displays an error message (Disk does not exist - Press any key to continue), and asks you to enter the disk name again.

Enter the directory where you want to install DECOMPXR: Disk:\

Directory where DECOMPXR will be installed: Disk:\Entry

Enter the name of the directory; <Enter> selects the default directory (\DCP).  INSTALL checks if the specified directory exists.  If not, it displays a warning message (Directory does not exist - Press C to confirm the creation).  Press the <C> key to validate the entry and to create the specified directory, as well as the EXAMPLES\ subdirectory, or any other key to modify the directory name.  If the specified directory exists INSTALL checks the existence of the EXAMPLES\ subdirectory and creates it if necessary.

DECOMPXR, auxiliary programs, and example files will be copied from drive A:

Please wait for the copy to be completed

INSTALL copies the various DECOMPXR program files on the specified disk and directory; example data files are copied in the EXAMPLES\ subdirectory.

Configuration section.  This part of the installation procedure asks you various informations about your X-ray diffraction system, as well as preferences for running DECOMPXR.  These parameters are stored in the DECOMP.CFG file that can be edited and modified with any text editor.  If you create or modify this file manually, please check the configuration file format described in Annex 1.  Most of these configuration parameters can be modified, but not saved, by using the Parameters menu when running DECOMPXR (refer to the Reference Guide section for more information).

CONFIGURATION PROCEDURE FOR X-RAY PATTERNS DECOMPOSITION PROGRAM
DECOMPXR - MARCH 1997 version

Default radiation (Cu, Co, Fe, Cr, Mo): Entry

Enter the radiation used on your diffraction system; allowed radiations are Cu, Co, Fe, Cr, and Mo.  There is no default entry.  INSTALL checks if the entry is one of the above; if not, it displays an error message (Invalid radiation - Press any key to continue), and asks you to enter the radiation again.

Please enter desired output level (3=None, 2=Minimum, 1=Total):

There is no default value.  A starting value of 2 is recommended.  See the Reference Guide section for more information about this parameter.  INSTALL checks if the entry is one of the above; if not it displays an error message (Invalid output level (1-3) - Press any key to continue), and prompts you to enter the desired output level again.

Path for temporary files - A virtual drive is recommended: Entry

The default entry is the directory where DECOMPXR has been installed.  See the Requirements section of this User's Guide for more information about the creation of a virtual drive on your computer.  INSTALL checks if the specified path exists; if not it displays a warning message (Warning: Path does not exist - Press C to continue, any other key to modify entry).  If you press the <C> key the specified path is written in the DECOMP.CFG file, but it is not created.  Consequently, you must create the specified virtual drive, or directory, before running DECOMPXR otherwise you will get an error message (Error in DECOMP.CFG file - Please correct it and run the program again) and DECOMPXR will stop; press any other key to modify the path.

Monochromatic radiation (0=No; 1=Yes)?

There is no default value.  Enter 0, except if your diffractometer is equipped with a monochromator that eliminates the K2 radiation, if you want to speed up the calculation (not recommended, but may be useful on a slow system when working on low-angle regions), or if you are working a lot on simulated diagrams.  See the Reference Guide section for information about this parameter.  INSTALL checks if the entry is one of the above; if not it displays an error message (Invalid entry (0/1) - Press any key to continue), and asks you to enter this parameter again.

Protection section.  This part of the installation procedure gives you a site-specific code.  Then, you will have to call E.R.M. to obtain the protection code specific to your computer.  This protection code will be written automatically in the DECOMP.CFG file.  You can install DECOMPXR on different computers, by asking E.R.M. the corresponding protection codes.

PROTECTION PROCEDURE FOR X-RAY PATTERNS DECOMPOSITION PROGRAM
DECOMPXR - MARCH 1997 version

Site specific code calculated by the installation procedure: Site-specific code

To know the code to be included in the DECOMP.CFG file, please note this eight characters code and contact either:

Dr. Alain Bouchet	Dr. Bruno Lanson
E.R.M. - Batiment GON	Environmental Geochemistry Group
Faculté des Sciences	LGIT - IRIGM

40 Avenue Recteur Pineau	BP 53
86022 Poitiers cedex	38041 Grenoble cedex 9
FRANCE	FRANCE
Phone# +33 (0)5 49 46 18 11	Phone# +33 (0)4 76 82 80 17

Fax# +33 (0)5 49 45 40 26	Fax# +33 (0)4 76 82 81 01

E-mail : Bruno.Lanson@obs.ujf-grenoble.fr

Please enter the protection code: Protection code

You can exit the installation procedure with Ctrl+Break

If you do so, you will need to run INSTDCP later to complete installation

If you hit <Ctrl>+<Break> you MUST run INSTDCP.EXE later from the directory where the DECOMP.CFG configuration file is located.  This latter program will execute only the protection section of this installation procedure.  The protection code given by your E.R.M. correspondent must be written in the DECOMP.CFG to complete the installation.

Installation completed - Type DECOMPXR to run the decomposition program

I hope you will enjoy it - Have a nice day

Remark: If you upgrade your operating system from DOS 3.nn to DOS 4.0 or a later version, the site specific code of your computer will change and you will have to modify the protection code by running the INSTDCP.EXE program.  No change is necessary if you upgrade from DOS version 4.nn to version 5.0 or later.

REFERENCE GUIDE

Remark: Not to reset the different parameters each time you run DECOMPXR, you may modify the DECOMP.CFG configuration file and adapt it to the configuration of your diffraction system.  This modification can be done with any text editor, but you have to respect the file format described in the Annex 1.

Remark: When you have to enter a value or a file name, etc., usually a default entry is displayed in a special window.  You can move within this window using the <Left arrow> (move one character to the left), the <Right arrow> (move one character to the right), the <Home> (move to the first character of the entry), and the <End> (move to the last character of the entry) keys.  If you want to delete a character, you can either use the <Backspace> key (delete one character to the left of the cursor) or the <Delete> (delete the character under the cursor) key; to delete the entire entry press the <Escape> key.  You add characters simply by typing them.  You can also note that if the first character you type is not one of the above key when you have to enter a value, the default entry is cancelled and you simply can type your entry.

Remark: DECOMPXR includes a control of most of the errors that may occur during the execution.  When an error message is displayed, you must press any key to clear it; then you can keep using the program.

File: Main menu for file-related operations.

Load: To load the data file you want to decompose.

Filter: This is used to display only selected files.  You can use it as in the DOS command DIR (e.g. *.DAT will display all the files with the extension DAT).  If you validate a blank entry, the Load action is cancelled and no new file is loaded.

Then you get a table with all matching names.  You can browse it with the four arrow keys, the <Home> and the <End> keys.

To select a file press the <Enter> key when it is highlighted.  You can select a work file from a different directory than the current one.  You just move from the current directory by selecting the desired directory.

The maximum number of data points in a given pattern is set to 2801.  If you try to load a data file containing more than 2801 data points, an error message will be displayed (Too many data points - Impossible to load this file) and no new data file is loaded.

To escape without selecting a data file press the <Escape> key; if a data file was already loaded it is kept in memory; if not, no data file is loaded.

Directory: This option enables you to change the default directory.

You have to enter the name of the desired directory.  The name is checked, and an error message is displayed if it is not valid.

If you don't want to change the default directory, just validate the default entry.

Edit: To display the selected data file, and to define the angular range you want to decompose.

You can zoom on the angular range you want to decompose.  To do so, press the mouse left button (MLB) to define one end of the zoom, hold it down while moving the mouse and release it to define the other limit of the zoom.  A red line is displayed on the X-Axis to check current selection.  This zoom defines the range for calculation (a range with half a degree 2 of near-zero values on each side is a good range).

To cancel the Zoom action just press the mouse right button (MRB).

To exit press the MLB in the End box.  If a new angular range has not been defined for the decomposition, the range of the last calculation is kept in memory.  If no decomposition has been performed so far the complete angular range will be decomposed (be aware of the 1001 data points limit for the angular range to fit).

Parameters: Main menu to set the various calculation parameters.

Range: To define the angular range to decompose.

The angular range you want to decompose can be defined either with the File+Edit option or with the present option.  If background stripping is to be performed before the calculation (Parameters+Pre-Treat+Yes option, see below) there is a third way to define the angular range to decompose, similar to the File+Edit option.

You have to enter the lower and upper limit values.  These values must be included in the global angular range, the upper limit must be larger than the lower one, and the defined range must contain less than 1001 data points (absolute maximum, not checked).

These lower and upper limit values are kept in memory even if you change the data file, which is a useful feature when you are treating an homogeneous series of X-ray patterns.

Elem. Curves: To define the number of elementary curves (from 1 to 6) to fit on the experimental pattern.  3 is the default value when the program starts.

Iterations: To define the maximum number of iterations to perform.

To limit calculation time, absolute maximum is 50 for the least squares method, and 5000 for the non-linear simplex method.  Usually, maximum precision is reached well before these values, thus default values are 25 for the least squares method, and 1500 for the non linear simplex method.

To avoid incomplete calculations and intermediate results, it is recommended always to run the calculation until maximum precision is reached (0.0005 °2 adjustments on both position and width, and 0.5 counts on intensity).

Output: To define the output level during calculation.

Total: Initial and calculated peak characteristics are printed as well as several important parameters related to the calculation (data file name, sample identification, angular range decomposed, number of iterations performed, reliability factor).  All intermediate results also are printed (calculated adjustments and corrected values) after each iteration.

Minimum: Default value set in DECOMP.CFG.  Initial and calculated peak characteristics are printed as well as several important parameters related to the calculation (data file name, sample identification, angular range decomposed, number of iterations performed, reliability factor).  Nothing is printed during calculation.

None: Nothing at all is printed.  This option is useful if there is no printer connected, or if you want to save paper during initial tries.

This parameter is set in the DECOMP.CFG file, but can be modified for a specific set of calculations.

K2: To define the presence of the K2 radiation.

The default value is set in DECOMP.CFG.  If the selected diagram was acquired or calculated with a different radiation type, this parameter can be changed either by modifying the DECOMP.CFG file or by using the Parameters+K2+K1 option.

Pre-Treat: To perform smoothing and/or background stripping on the selected X-ray pattern.

DECOMPXR assumes that there is no background (i.e. that the background is horizontal with a zero value, the background is not adjusted during calculation).  Therefore, it is necessary to subtract this background before running the iterative fit routine.  In order not to have to perform this background stripping each time you decompose a specific profile, the experimental pattern without background can be saved on disk.

"Save treated patterns (Y)/N: " The default value <Y> is to save treated patterns. Press the <N> key if the pattern without background is not to be saved.

"Window for smoothing:   4": This value is the number of points taken into account on each side of a considered point to perform the free cubic spline smoothing procedure (see Lanson, 1990, for details).  This value may vary between 0 and 12; the default value is 4 and gives good results in most cases; however, if the treated pattern is free of experimental noise, or if the experimental noise is very low, this value can be reduced to a value lower than 3, and no smoothing will be performed.  On the other hand, if the pattern quality is poor, this value may be increased to remove most of the experimental noise.  However, one must remember that the decomposition treatment is a very accurate method, and that it may not be relevant to decompose a very poor pattern.  One should also be aware of line profile alteration that may results from data smoothing.

This pre-treatment option is disabled after the first calculation, not to perform the smoothing and background stripping procedures twice on the same pattern.

Radiation: To define the radiation type.

The default radiation is set in the DECOMP.CFG file, but can be modified for a specific set of calculations (available radiations are Cu, Co, Fe, Cr, and Mo).

Simulation: To configure DECOMPXR for simulated pattern decomposition.

Simulated patterns are calculated with a monochromatic wavelength, and there is no experimental noise; thus when this option is enabled, i) the radiation is supposed to be monochromatic and ii) there is no correction on intensities during background calculation (for details on the background subtraction algorithm used, see Lanson, 1990).

Batch Calc.: To perform extensive calculation using command files.

The use of these options is detailed in the Go option; the format of the command files is described in Annex 1.

No: General case, the user defines for each data file, the angular range to decompose and the initial values set.

Yes: To perform several consecutive decompositions without any intervention of the user.  A command file is used in which the names of the various files to decompose, as well as the angular ranges to decompose and the initial values sets are stored.

Bkg: To perform several consecutive background subtractions.  A command file is used in which are stored the names of the various files to treat.

As a first step, it is much more educational to "practice", and to understand the "behavior" of DECOMPXR and the procedure to follow when dealing with complex polyphasic clay samples, before running extensive calculations.

However, for the experienced user, these options are useful making it possible to run numerous calculations, especially time-consuming ones, at night or during week-ends.

Save Results: To save decomposition results on disk.

If this option is enabled, after each decomposition two result files are saved on disk.  The first file name is filename.DCn, where filename is the name of the decomposed data file, and n is the number of elementary curves fitted on this profile.  The format of these result files is detailed in Annex 1.  These result files may be used to define an initial values set for a latter calculation (see Go option).

An additional result file filename.FIT is saved on disk.  In the first column, this ASCII file contains the measured intensities; the adjusted global intensities are given in the second column.  The intensities of the various elementary peaks are given in the following columns.

Go: To perform the decomposition.

Routine procedure: If no automatic calculation option has been activated (Parameters+Batch Calc.+No option), DECOMPXR first checks if a data file has been loaded (File+Load option) previously.  If not, an error message is displayed: No data file - Calculation aborted.  A data file must be loaded in memory to run the decomposition procedure, even if you plan on using previously stored values (see below).

If the pre-treatment is enabled (Parameters+Pre-Treat+Yes option), the experimental pattern is first smoothed (smoothing skipped if the window is less than 3 points wide, see Parameters+Pre-Treat); then a default background is calculated automatically.  Both the experimental pattern and the calculated background are displayed, a zoom is available on both X- and Y-axes (not only on the X-axis).  This zoom works as in the File+Edit option, press the mouse left button (MLB) to define one corner of the zoom, hold MLB down while moving the mouse and release it to define the opposite corner of the zoom.  A red line is displayed on both X- and Y-axes to check current selection.  Click the mouse right button to reset the zoom.

The default background calculation gives good and reproducible results.  However, it is possible to modify the default background by editing the points defining the background which are figured by small white-colored boxes.  Once the range where the background is to be modified has been expanded, the Edit mode must be activated by clicking the MLB in the Edit box; the color of this box is switched to red, and you can set (MLB) or remove (MRB) points defining the background.

By clicking the MRB, the nearest point displayed is removed; its color is switched from white to magenta; by clicking the MLB, a point is set at the arrow point location (X and Y values).  Points may not be set or removed outside the angular and the intensity ranges displayed; moreover, these points are not figured outside of the XY range displayed.

When you are done with the modifications, the Edit mode must be disabled by clicking the MLB in the Edit box (the color of this box is switched back to cyan), and the new background calculated by clicking the MLB in the Calc. box.  The new background is displayed then; it is possible to modify it again using the same procedure, or to validate it by clicking the MLB in the Valid box.

Remark: The validated background is the last background calculated, thus if background points are edited but no calculation is performed after the modifications, these are not taken into account.

Remark: It is possible to reduce the angular range taken into account to calculate the background.  The Range mode must be activated by clicking the MLB in the Range box; the color of this box is switched to red, all other boxes are removed, and you can define the angular range using an X-axis zoom procedure similar to the File+Edit option.  When you are done, the Range mode must be disabled by clicking the MLB in the Range box (the color of this box is switched back to cyan, and the other boxes are displayed again), and a new default background is calculated and displayed.  All the usual options are available on this background calculated on a restricted angular range.  It is recommended to save the restricted range as a distinct file.

Remark: If the background had been subtracted already, or if you want to save a reduced range with the background as a separate file, you can click MLB the Cancel box.  No background subtraction is performed, and the pre-treatment procedure will proceed.

Then, the calculated background is subtracted, and the pattern without background is saved if this option has not been disabled previously.  If the name entered to save the experimental pattern without background matches an already existing file name a warning message (File already exists. Overwrite? Y/(N):) is displayed.  If the <Y> key is not pressed, the file name has to be entered again.

The pattern without background is displayed and it is possible to define the angular range you want to consider for the decomposition procedure (option identical to the File+Edit option).  If no angular range is defined, the previous range is kept.

Then (directly if the Parameters+Pre-Treat option is not enabled) the angular range to decompose is checked; if it does contain more than 1001 data points, the error message Angular range too large - Calculation aborted is displayed, the calculation is stopped, and DECOMPXR goes back to the main menu.

If not, the message Use previously stored initial values (*.DC* files) Y/(N): is displayed.

If the <Y> key is pressed all the files matching the selected filter (default *.DC*) are displayed in a table similar to the one displayed by the File+Load option (see above).

These files contain the name of the data file to decompose, the angular range to fit, the number of elementary curves to adjust, and the initial values set (check the structure of a result file, Annex 1).  As the data file to decompose is changed when using this option, if you want to decompose a specific file with the initial values from another file (useful in a series of closely related patterns), the initial values set is to be loaded at first by using this option, and then the desired data file may be selected with the File+Load option.

If the data file name read in the selected *.DC* file does not match an existing file name, the error message Error file file name.DAT - No such file is displayed; if a blank filter is validated, or if the <Escape> key is pressed while browsing the file name table, no *.DC* file is loaded.  In both cases, the data file already loaded is kept in memory, as well as the previous initial values set if it exists.

For the first calculation of the session, initial values have to be entered for any elementary curve.  Position, intensity, width, and shape for the K1 contribution must be set, as well as the shape for the K2 contribution if the radiation is not monochromatic; position, width, and intensity values for the K2 contribution depend on the K1 contribution values and are calculated.  Positions and widths are given in °2, intensities in total counts, the shape is either L for Lorentzian (Cauchy) or G for Gaussian.

If you want to modify an option (e.g. change the number of elementary curves to fit) before setting initial values you have to press the <Escape> key at first to exit the empty initial values table.  If you don't press it at first, you can exit only after initialization of all the values.

Once all the initial values have been entered, if it is not the first calculation of the session, or if previously stored initial values have been used, values are present in the initial values table.  You can move within this table with the four arrow keys, the <Home>, <End>, <Page up>, and <Page down> keys.  To edit a value place the cursor on the cell to be modified, press the <F1> key, and modify the existing value or simply type the new one.

By pressing the <F3> key, the whole table is cleared; and initial values have to be entered for any elementary curve, as for the first calculation of the session.  However, if the <Escape> key is pressed at first all the existing initial values set is saved and displayed when running the next calculation.

The <F4> key is used to save the initial parameters on disk for a latter calculation or to be part of a command file for night calculation (see below).  The name of the saved file is filename.DCn; filename is the name of the decomposed data file, n is the number of elementary curves fitted on this profile.  The format of these files is detailed in Annex 1.

The <F5> key allows a preview of the initial fit without performing any calculation; this option permits to modify "manually" the initial values set to get a faster convergence; however, one can note that the results do not depend on the initial values set, and that the computation time will be the only modified parameter.

The <F6> key allows one to fix the initial values set for one , or more, elementary peak.  The color of the fixed peak characteristics is switched from yellow to magenta, but may be edited; these characteristics are not adjusted during the iterative fitting procedure, the adjustement is performed only on the characteristics of the other elementary peaks.  By pressing again the <F6> key the characteristics of a fixed elementary peak are allowed to vary again for the next calculation.  This option is especially useful when the number of elementary peaks is high, and consequently when the maximum precision criterion is hard to reach.  By fixing the characteristics of few well characterized, and about unvarying, peaks it is possible to obtain a faster convergence on the other peak characteristics.  As a final step, the fixed peak characteristics have to be relaxed to reach the maximum precision criterion for the global fit.

The <F2> key validates the initial values set displayed and performs the calculation by running the least squares method:

i) The calculation reaches maximum precision (0.0005 °2 adjustments on both position and width, and 0.5 counts on intensity); the calculation is stopped, results are printed as a function of the output level (Parameters+Output option).  The optimum fit is displayed on the screen and may be printed on your printer by using the <Print Screen> key and a hardcopy utility. This utility is memory resident and must be loaded before running DECOMPXR; be sure that this utility is adapted to your graphic screen configuration.  When using the Output+Minimum parameter the results are printed on the upper half page, and the lower one may be used to print the calculated fit.

ii) The calculation gives a correct fit but maximum precision can not be reached.  The first verification is to run the least squares method for 25 or 50 more iterations to check that maximum precision cannot be reached.  Then, it may be interesting to use a larger angular range, or to print intermediate results (Parameters+Output+Total option) because calculated values may oscillate between two sets of values, usually very similar.  By resetting initial values in between the two sets, it is usually possible to reach maximum precision after a few tries.

iii) The calculation diverges with the least squares method or a singular matrix is detected; a message is diplayed: Divergence or Singular matrix - Program halted - Do you want to try with the non linear simplex method? (Y)/N, there are two options as a function of your answer:

- Enter a new set of initial values and try again with the least squares method.

- Perform the non linear simplex method with the same initial values (default answer).  Usually this method does not diverge but is much more time consuming; the mean number of iterations to reach maximum precision is about 1500 with three elementary peaks, instead of 15 with the least squares method, even though an iteration takes less time with the non linear simplex method than with the least squares method.  Consequently the use of this method should be limited to patterns that can not be fitted with the least squares method, e.g. when various phases with similar characteristics (position and width) are present in the same sample.  One can also note that with this method the results slightly depend on the initial values (Howard and Snyder, 1983).  Thus, it is important to carefully define these values (which can be done quickly when treating a series of homogeneous samples) and/or to run the decomposition program again with the initial values being the results of the previous calculation to check their stability.

Parameters+Batch Calc.+Yes: If the automatic calculation option has been activated (Parameters+Batch Calc.+Yes option), DECOMPXR first asks for the command file name; the default command file name is COMMAND.DCP, i.e. the default file name generated by LIST; if no extension is defined, the extension .DCP is added by default.

These command files may be generated with LIST (DECOMP option); they contain the names (see Annex 1 for the file format) of the various result files matching the selected filter (e.g. *.DC*), the result files containing the name of the data file to decompose, the angular range to fit, the number of elementary curves to adjust, and the initial values set (check the structure of a result file, Annex 1).

If the command file you specified is not present in the current directory, an error message is displayed: No such file.  Then the command file is verified; the following error messages may be displayed if the command file is not conform:.Error: Invalid number of files / Error file file name.DAT - No such file / Error file file name.DAT - Invalid range / Error file file name.DAT - Invalid number of elementary curves / Error file file name.DAT - Invalid initial values.  Please refer to the error messages section for details.

If no error is found, the various calculations corresponding to the *.DC* result files listed in the command file are performed, results are printed as a function of the output level (Parameters+Output option), and the optimum fit is copied on your printer automatically.  If the calculation performed with the least squares method diverges, the non linear simplex method is automatically performed.

Parameters+Batch Calc.+Bkg: If the automatic background stripping option has been activated (Parameters+Batch Calc.+Bkg option), DECOMPXR first asks for the command file name; the default command file name is COMMAND.LIB, i.e. the default file name generated by LIST; if no extension is defined, the extension .LIB is added by default.

These command files may be generated with LIST (DECOMP option); they contain the names (see Annex 1 for the file format) of the various data files matching the selected filter (e.g. *.DAT).

If the command file you specified is not present in the current directory, an error message is displayed: No such file.  Then the command file is verified; if one of the file names present in the command file does not match the name of an existing file, the following error message is displayed: Error file file name.DAT - No such file.

If no error is found, the background is calculated with the current parameters (Parameters+Simulation option) and automatically subtracted from the pattern.  The original data file is then overwritten with the pattern without background; be sure to get a backup version of these original data files.

This option is especially useful to subtract the background from numerous simulated pattern; its use is not recommended for experimental patterns, because of the experimental noise that may always induce some numerical aberration in the background calculation.

Quit: End of the session.  Return to DOS.

TROUBLESHOOTING - ERROR MESSAGES

A very common problem occurs when trying to print the calculated fit displayed on the screen by using the <Print Screen> key.  If the graphics does not print at all, check that your hardcopy utility (such as GRAPHICS.COM, PJSCREEN.EXE, among many others) has been loaded before running DECOMPXR.  If the graphics is not printed correctly, make sure that this utility is adapted to your graphic screen configuration, some of these utilities do not handle the hardcopy of VGA screens.  When using the Output+Minimum parameter the results are printed on the upper half page, and the lower one may be used to print the calculated fit.

Cannot find the DECOMP.CFG file - Please copy it in the default directory and run the program again: The configuration file DECOMP.CFG must be present in the directory you run DECOMPXR from.  Fatal error.

DECOMPXR incorrectly installed - Program stopped - Please run INSTDCP and modify the protection code in DECOMP.CFG: The protection code in DECOMP.CFG is incorrect, please contact E.R.M. with your site-specific code available to obtain the corresponding protection code.  Fatal error.

Error in file DECOMP.CFG - Please correct it and run the program again: Check that the structure of the DECOMP.CFG file is conform to the format presented in this User's Guide.  Fatal error.

Too many data points - Impossible to load this file:  More than 2801 data points are present in the selected data file.  This is not allowed by DECOMPXR; reduce the angular range covered by this file, or use a larger step size (either by running again this sample, or by editing the present data file).  Warning.

No such directory: The directory you specified as current directory is not a valid directory.  Check the arborescence of your disk.  Warning.

Error nnn with graphic card - Initialization impossible - Program halted: An error occurred preventing the initialization of the graphic mode.  No graphic can be displayed.  Fatal error.

The following errors may occur:

nnn = -2: No graphic card has been detected.  Check the configuration of your computer, especially that your graphic card is working correctly.

nnn = -3: Graphic driver not found.  The graphic card in use is not a high definition color EGA or VGA card as required; modify the configuration of your system or contact your dealer to obtain the valid driver.

nnn = -4: Invalid graphic driver.  The graphic card in use is not a high definition color EGA or VGA card as required; modify the configuration of your system or contact your dealer to obtain a valid driver.

nnn = -5: Not enough memory to load the graphic driver.  The graphic card in use is not a high definition color EGA or VGA card as required; unload all the resident programs you are not using at present and try again.  Make sure that your virtual drive is defined in the extended memory (switch /E).  Modify the configuration of your system or contact your dealer.

Mouse not installed - Option not available: i) You tried to edit (File+Edit option) the selected data file but a mouse has not been detected.  Check that your mouse is correctly connected and that your mouse driver is correctly installed.  ii) You tried to eliminate the background (Parameters+Pre-treat+Yes option and then Go) of your file but a mouse has not been detected.  Check that your mouse is correctly connected and that your mouse driver is correctly installed.  The automatic background stripping does not require the presence of a mouse but it is not possible to visualize the calculated backgrounds.  Warning.

No work file - impossible to define a range: You selected the Parameters+Range option, but no data file has been selected.  A data file must be selected (File+Load) before defining the angular range you want to decompose.  Warning.

No data file - Calculation aborted: No data file has been loaded (File+Load option) before running the decomposition procedure (Go option).  A data file must be loaded even if you plan on using previously saved initial values.  Warning.

No data file - Option not available: No data file has been loaded (File+Load option) before running the File+Edit option.  Warning.

Angular range too large - Calculation aborted: More than 1001 data points are present in the selected angular range.  This is not allowed by DECOMPXR, define a smaller angular range, or use a larger step size (either running again this sample, or editing the present data file).  Warning.

Runtime error 152 at nnnn:dddd: The output level is set to total or minimum (Parameters+Output+Total or Parameters+Output+Minimum option), and no printer is connected, the connected printer is not turned on, or the connected printer is not on line.  Thus, a fatal error occurred when you run the decomposition procedure (Go option + <F2> to validate the initial values set).  Fatal error.

Runtime error 201 at nnnn:dddd: The data file you tried to load actually contains more than 2801 data points, even though the number of data points read on the first line of this file (see Annex 1 for the file format) is lower or equal to 2801.  Fatal error.

No such file: i) There is no file matching the filter of the File+Load option, and no sub-directories on the current disk.  ii) You have selected the automatic calculation option (Parameters+Batch Calc.+Yes) but  the command file you specified is not present in the current directory; check the presence of this file.  iii) You have selected the automatic background stripping option (Parameters+Batch Calc.+Bkg) but  the command file you specified is not present in the current directory; check the presence of this file.  Warning.

Error: Invalid number of files: You have selected the automatic calculation option (Parameters+Batch Calc.+Yes) but the command file you specified is not valid; run LIST (DECOMP option) to create a valid command file.  Warning.

Error file file name.DAT - No such file: i) You have selected the automatic calculation option (Parameters+Batch Calc.+Yes) but the data file defined in the file file name.DC? is not present in the current directory; check the structure of this file file name.DC?, and the presence of the specified data.file.  ii)  You tried to run the program using previously stored initial values (*.DC* files), but the data file defined in the file file name.DC? you specified is not present in the current directory; check the structure of this file file name.DC?, and the presence of the specified data.file.  Warning.

Error file file name.DAT - Invalid range: You have selected the automatic calculation option (Parameters+Batch Calc.+Yes) but the calculation range defined in the file file name.DC? is not valid; check the structure of this file, and the angular range limits (e.g. the upper limit must be larger than the lower one).  Warning.

Error file file name.DAT - Invalid number of elementary curves: You have selected the automatic calculation option (Parameters+Batch Calc.+Yes) but the number of elementary curves defined in the file file name.DC? is not valid; check the structure of this file, and the number of elementary curves which must be from 1 to 6.  Warning.

Error file file name.DAT - Invalid initial values: You have selected the automatic calculation option (Parameters+Batch Calc.+Yes) but the initial values set defined in the file file name.DC? is not valid; check the structure of this file, and the initial values set (e.g. positions of the various elementary peaks must be located within the angular range, their intensities and widths must be positive values, and their shapes must be either L or G).  Warning.

If you need help or further informations you can contact:

Dr. Alain Bouchet	Dr. Bruno Lanson
E.R.M. - Batiment GON	Environmental Geochemistry Group
Faculté des Sciences	LGIT - IRIGM

40 Avenue Recteur Pineau	BP 53
86022 Poitiers cedex	38041 Grenoble cedex 9
FRANCE	FRANCE

Phone# +33 (0)5 49 46 18 11	Phone# +33 (0)4 76 82 80 17

Fax# +33 (0)5 49 45 40 26	Fax# +33 (0)4 76 82 81 01

E-mail : Bruno.Lanson@obs.ujf-grenoble.fr

AUXILIARY PROGRAMS

These programs are meant to provide the user with additional utilities to increase DECOMPXR's ease-of-use, especially by executing automatically series of repeated actions, such as file format translations.  It is also possible to link automatically a series of time-consuming calculations (non linear simplex method) and to make then run at night or during week-ends.  All these programs have been conceived to minimize the user's presence in front of his screen.

COMMAND FILE GENERATION

LIST.EXE: This program is used to create command files, which can be used either by DECOMPXR or by programs translating data files from various formats to the format used by DECOMPXR.  The formats for both uses are different and described in the Annex 1, but the procedure to create these files is exactly the same in both cases:

You are first prompted for the command file name, a default file name being proposed as a function of the command file type you are creating; the file name can be edited as any entry in DECOMPXR.  If no extension is defined, a default extension is added.  Then a default filter is proposed and may be edited; all the files which name matches the selected filter are stored in the command file to be treated.

Transcod: To create command files for format translators.

Default command file name: COMMAND.DAT; default extension for the command file: .DAT; default filter for file selection: *.RAW.

Decomp: To create command files for DECOMPXR.

Automatic calculation: Command files for Parameters+Batch Calc.+Yes option.

Default command file name: COMMAND.DCP; default extension: .DCP; default filter: *.DC*.

Automatic background stripping: Command files for Parameters+Batch Calc.+Bkg option.  Default command file name: COMMAND.LIB; default extension: .LIB; default filter: *.DAT.

FORMAT TRANSLATOR PROGRAMS

These programs translate automatically the files listed in a command file from their original format to the format used by DECOMPXR.  The original data files are not deleted after the format translation.  When running these programs, you are prompted to enter the name of the command file containing the names of the raw data files to be translated; the default command file name is COMMAND.DAT and can be validated by hitting the <Return> key.  In the command file, the names of the data files to be translated are stored without any extension; a default extension is added by the format translator used.  The command file name is checked; if the specified file name does not exist, an error message (Command file does not exist - Program stopped - Press any key to continue) is displayed and the program is stopped.  When a data file to be translated is missing, an error message is displayed (Runtime error 002 at nnnn:nnnn), and the program is stopped.

The data file format used by DECOMPXR is described in the Annex 1.

RAW1DAT.EXE: This program is used to translate raw data files generated by the Diffrac-AT diffraction management system (up to version 3.0 excluded) to the format used by DECOMPXR.  A default .DAT extension is added to the command file name, if the extension is omitted; if the specified command file does not exist the error message displayed is Error, missing command file - Error at line 24 in RAWDAT - Error number 3012.  The default extension for the data files to be translated is .RAW.  When a data file is missing, a warning message (Unable to open file file name) is displayed, and the program proceeds with the next file.  If the raw data file format is the Diffrac-AT version 3.0 format, the data file is not translated but no error message is displayed.

RAW2DAT.EXE: This program is used to translate raw data files generated by the Diffrac-AT diffraction management system (version 3.0 and more recent) to the format used by DECOMPXR.  The default extension for the data files to be translated is .RAW.  The translation from the RAW format to the DECOMPXR format is performed as a two-step procedure.  The first step (translation from the RAW format to the UXD universal ASCII file format) is performed by Socabim's program XCH.EXE provided with the DIFFRAC-AT package; as a second step, the .UXD ASCII files are translated to the format used by DECOMPXR, and then deleted.  This later part is performed with a program similar to UXDDAT (see below).

UXDDAT.EXE: This program is used to translate the ASCII data files using the "universal" ASCII file format proposed by Socabim to the format used by DECOMPXR.  The default extension for the data files to be translated is .UXD.  If the .UXD data file includes two different angular ranges, only the first one is converted and stored with the DECOMPXR format; no warning message is displayed.  At present, UXDDAT is unable to translate 2THETACOUNTSTIME data file format (see DIFFRAC-AT user's guide for details), and D-I peak list.  If such a file is listed in the command file, an error message (Error translating filename.UXD - UXDDAT does not handle 2THETACOUNTSTIME data file format / UXDDAT cannot translate D-I lists - Press any key to continue) is displayed, the file is skipped, and the program proceeds with the next file to be translated.  UXDDAT also checks the consistency between the START value and the starting angle; if a problem is detected, an error message (Error translating filename.UXD - Inconsistency between the START value and the starting angle - Press any key to continue) is displayed, the file is skipped, and the program proceeds with the next file to be translated.

APDDAT.EXE: This program is used to translate raw data files generated by Philips APD diffraction management system running on IBM-compatible computers to the format used by DECOMPXR.  The default extension for the data files to be translated is .RD.

SMSDAT.EXE: This program is used to translate ASCII data files generated by the Siemens D5000 diffraction management system running on Vax computers to the format used by DECOMPXR after transfer of these files on an IBM-compatible computer.  The default extension for the data files to be translated is .DAT.  These raw data files are renamed with a .SMS extension during format translation; if a .SMS file already exists, an error message is displayed (Runtime error 005 at nnnn:nnnn), and the program is stopped.  An error message (Runtime error xxx at nnnn:nnnn, with 200xxx) is displayed also if the header format of the original data file does not match the following model:

1275 M SONDAGE M PARIS BASIN PARIS  EG: sample name on the first line.

07-APR-91: measurement date, or comments, on the second line.

2.0000 50.0000 0.0200 3.0000: start angle, end angle, step size, and counting time on the third line.

9556. 9428. 9126. 9076. 8809. 8392. 8389. 8084. 8007. 8029.: raw intensities.

TRUDAT.EXE: This program is used to translate data files generated by the NEWMOD® program to the format used by DECOMPXR.  The default extension for the data files to be translated is .TRU.

EXAMPLES

Introduction: Three data files (15WS7115.DAT, CHV300A1.DAT, and CHV16451.DAT) are provided with the decomposition procedure used to obtain their decompositions.  These three examples are meant to help you understand the procedure to follow when dealing with complex polyphasic clay samples and to give you the few tips that make decomposition an easy-to-use tool.  These data files are copied in the EXAMPLES\ subdirectory by the installation procedure, and you just have to follow the step-by-step procedure proposed.  The use of the program itself is not detailed in this example section because of its simplicity; however, the name of the different options used are given (underlined) to make it easy to find the necessary information in the reference guide if you get any problem.  Peak characteristics (initial and adjusted values) are given only for the K1 component because the characteristics of the K2 component are calculated as a function of the first ones.  The example profiles were obtained with a Cu K(1+2) radiation; if your configuration (DECOMP.CFG file) is different, you will have to set these parameters to the right values (Parameters+Radiation+Cu and Parameters+K1+K2+K1+K2) to follow these examples.

The first example is performed on the file 15WS7115.DAT.  You first load the data file (File+Load option); the pattern can be displayed (File+Edit option - Figure Exp.1).  One can observe that the pattern shows a little experimental noise; therefore, a data smoothing procedure is useful before background stripping (Parameters+Pre-Treat+Yes option - Window for smoothing : 4).  There are no other parameters to change at that point and you can run the calculation procedure (Go option).  The calculated background is reasonable (Figure Exp.2), and you can validate it (Click MLB in the Valid box), and possibly save the treated pattern (Figure Exp.3).  You can define the angular range to decompose from 6.5 to 10.5 °2, including about half a degree 2 of near-zero values on each side of the considered range (procedure similar to the File+Edit option - Figure Exp.4).

On this expanded diagram, one can get a good approximation of the initial values for the calculation.  Obviously, two peaks are necessary to fit this profile: i) a major one at 8.2 °2, about 2/3x4090 counts in intensity for K1 (total intensity is K1+K2, the K2 contribution intensity being half of the K1 component), quite sharp.  ii) another sharp peak, located at about 9.1 °2, and basically three times less intense than the previous one.  One can summarized these observations with the following initial values:

Position	Intensity	FWHM	Shape K1	Shape K2

8.20	2500	0.5	G	G

9.10	800	0.5	G	G

As the default number of elementary curves is 3, when you first see the initialization screen, you get 3 empty columns; press the <Escape> key to exit the calculation procedure, and set the number of elementary curves to 2 (Parameters+Elem. Curves+2 option).  Then you can run the calculation procedure again (Go option).

Remark: The shapes of both K1 and K2 contributions are assumed to be gaussian; despite opposite statements in the litterature (Howard and Preston, 1989; Stern et al., 1991), our experience is that most of the natural peaks can be fitted with Gaussian and/or Lorentzian elementary peaks (Lanson, 1990; Lanson and Champion, 1991; Lanson and Besson, 1992; Lanson and Velde, 1992; Bouchet et al., 1992 among others).  When the fit is not good with these shapes, the problem is generally not the peak shape, but the number of elementary peaks used to fit the profile, and therefore the number of phases present in the sample.  One should not forget that Gaussian and Lorentzian shapes are specific cases of the Pearson 7 shape (respectively when the exponent is maximized and when the exponent is set to 1.0); using this profile shape will improve the fit by increasing the number of adjustable parameters (4 for Pearson 7 instead of 3 for Gauss or Lorentz).

A preview (<F5>), allows one to check that these initial values are not aberrant (Figure Exp.5) before performing the calculation.  Maximum precision is reached after 8 iterations, the results are as follow:

Position	Intensity	FWHM	Shape K1	Shape K2

8.281	2647	0.883	Gaussian	Gaussian

9.193	785	0.619	Gaussian	Gaussian

The resulting fit (Figure Exp.6) is good; both peaks being related to an illite/smectite ordered (R3) mixed-layered (Lanson and Besson, 1992).

The second example is performed on the file CHV350A1.DAT.  One can observe that the pattern (Figure Exp.7) is intense and shows very little experimental noise; therefore, a data smoothing procedure is not necessary before background stripping (Parameters+Pre-Treat+Yes option - Window for smoothing : 0).  There are no other parameters to change at that point and you can run the calculation procedure.  The calculated background is reasonable (Figure Exp.8); on the treated pattern (Figure Exp.9), you define the angular range to decompose from 4.6 to 10.4 °2 (Figure Exp.10).

On this expanded diagram, one can get a good approximation of the initial values for the calculation.  Obviously, at least three peaks are necessary to fit such a profile: i) a major one at 8.8 °2, very intense, and quite sharp.  ii) another sharp peak, located at 6.25 °2, and basically twice less intense than the previous one.  iii) a very broad peak located between 7.5 and 8.0 °2, slightly more intense than the previous one.  One can summarize these observations with the following initial values:

Position	Intensity	FWHM	Shape K1	Shape K2

8.80	12000	0.4	G	G

7.75	7500	2.0	G	G

6.25	5000	0.65	G	G

A preview (<F5>), allows one to check that these initial values are not aberrant (Figure Exp.11) before performing the calculation.  Maximum precision is reached after 11 iterations, the results are as follow:

Position	Intensity	FWHM	Shape K1	Shape K2

8.776	8181	0.590	Gaussian	Gaussian

8.008	9663	1.793	Gaussian	Gaussian

6.313	4495	1.058	Gaussian	Gaussian

The resulting fit (Figure Exp.12) is not satisfying, especially the low-angle shoulder of the 8.8 °2 peak.  As this is the best fit possible with three elementary peaks, it is necessary to introduce an additional peak to improve the fit of this profile.  The sharp 8.8 °2 peak is split in two components, a very sharp one to model the maximum, and a broader contribution (width unchanged from the previous calculation).  The other peak characteristics remain unchanged from the last calculation, to run this calculation.  Consequently, the initial values are as follow (Preview on Figure Exp.13):

Position	Intensity	FWHM	Shape K1	Shape K2

8.85	2500	0.35	Gaussian	Gaussian

8.70	6500	0.590	Gaussian	Gaussian

8.008	9663	1.793	Gaussian	Gaussian

6.313	4495	1.058	Gaussian	Gaussian

Maximum precision is reached after 18 iterations, and the final values are:

Position	Intensity	FWHM	Shape K1	Shape K2

8.824	4784	0.344	Gaussian	Gaussian

8.635	7048	1.010	Gaussian	Gaussian

7.687	7765	1.946	Gaussian	Gaussian

6.201	3696	0.779	Gaussian	Gaussian

On Figure Exp.14, one can see that the fit is good; the only problem is the adjustment of the 6.2 °2 peak which is not sharp enough and does not get a wide enough tail on the fitted pattern.  To improve this, it is possible to alter the shape of this peak, introducing a Lorentzian component either for the K1 or the K2 contributions or for both.  The following results were obtained after 9 additional iterations, just by changing the shape of the K2 contribution of the 6.2 °2 peak to Lorentzian; all the other peak characteristics were kept unchanged from the previous calculation to perform this one:

Position	Intensity	FWHM	Shape K1	Shape K2

8.823	4741	0.341	Gaussian	Gaussian

8.640	7259	1.016	Gaussian	Gaussian

7.687	7663	1.888	Gaussian	Gaussian

6.212	3965	0.764	Gaussian	Lorentzian

On figure Exp.15, the fit has been improved slightly and can be considered as good, even though it is probably possible to get a more perfect fit by changing again the shape of the 6.2 °2 peak.

The next step is the identification; in the present case the various elementary peaks were respectively associated to a detrital (?) mica, an authigenic illite, an illite/smectite ordered mixed-layered, and a chlorite (Lanson and Champion, 1991; Lanson and Besson, 1992).

When you start using DECOMPXR the most difficult problem is to estimate the width (FWHM) of the various elementary peaks to define initial values for calculations.  However, DECOMPXR allows a very important adjustment variations from the initial values and these do not have to be determined accurately.  In the initialization procedure for the previous example, it is possible to set all the FWHM to the same 0.5 °2 value, which is obviously very far from optimal values (Preview on Figure Exp.16):

Position	Intensity	FWHM	Shape K1	Shape K2

8.80	12000	0.5	Gaussian	Gaussian

7.75	7500	0.5	Gaussian	Gaussian

6.25	5000	0.5	Gaussian	Gaussian

Maximum precision is reached slower than with initial values more precisely defined (17 iterations instead of 11) but the results are identical (Figure Exp.17 and Figure Exp.12).

The third pattern, CHV16451.DAT (Figure Exp. 18), is very intense and there is no experimental noise.  Therefore, as for the previous example no smoothing is necessary (Parameters+PreTreat+Yes option, Window for smoothing = 0), before stripping the background.  The background calculated automatically can be accepted, the angular range to decompose is set from 4.5 to 10.5 °2 (Figure Exp.19).  At least three peaks are necessary to fit this profile, because of the pronounced asymmetry of the 8.8 °2 peak.  A strong sharp peak at 8.8 °2, a peak around 8.0 °2, fairly intense and quite broad to model the low-angle shoulder, and finally a weak 6.25 °2 peak are assumed as initial values for the first calculation (see preview on Figure Exp.20):

Position	Intensity	FWHM	Shape K1	Shape K2

8.80	20000	0.5	Gaussian	Gaussian

8.10	7500	1.0	Gaussian	Gaussian

6.25	2200	0.65	Gaussian	Lorentzian

The shape of the K2 component of the 6.25 °2 peak is assumed to be lorentzian because of the results obtained on the previous example.  Maximum precision is reached after 19 iterations, and the following values are calculated:

Position	Intensity	FWHM	Shape K1	Shape K2

8.795	10602	0.365	Gaussian	Gaussian

8.459	14204	1.050	Gaussian	Gaussian

7.252	2705	2.284	Gaussian	Lorentzian

On Figure Exp.21, it is obvious that the fit is not satisfying; the elementary peak initialized to fit the 6.25 °2 peak shifted to 7.25 °2,which means that it is not possible to fit the 8.8 °2 band and its shoulder with only two elementary peaks.  An additional peak is to be introduced to fit accurately this band.  Without any information, a strong sharp peak is assumed, as well as two peaks with similar intensities, but with position and FWHM slightly contrasted which give a reasonable initial adjustment (Figure Exp.22):

Position	Intensity	FWHM	Shape K1	Shape K2

8.80	14000	0.35	Gaussian	Gaussian

8.50	7500	0.75	Gaussian	Gaussian

8.10	7500	1.25	Gaussian	Gaussian

6.25	2200	0.65	Gaussian	Lorentzian

Maximum precision is reached after 14 iterations:

Position	Intensity	FWHM	Shape K1	Shape K2

8.822	8408	0.288	Gaussian	Gaussian

8.608	9762	0.718	Gaussian	Gaussian

8.245	9091	1.447	Gaussian	Gaussian

6.385	2224	1.027	Gaussian	Lorentzian

The fit presented on Figure Exp.23 is good, and the identification similar to the previous sample.

As in the previous example, the difficult part of the decomposition treatment is to fit the asymmetrical 8.8 °2 peak and its shoulder.  To make this easier, it is possible in a first approximation not to consider the complete angular range, and especially not to consider the 6.2 °2 peak.  A restricted angular range can be defined from 7 to 10.5 °2 (Figure Exp.24).  As discussed above (Figure Exp.21), three peaks are necessary to fit the 8.8 °2 peak and its shoulder.  Initial values are set as follow:

Position	Intensity	FWHM	Shape K1	Shape K2

8.80	10000	0.30	Gaussian	Gaussian

8.40	12000	0.80	Gaussian	Gaussian

8.00	8000	1.20	Gaussian	Gaussian

The initial fit is not very good (Preview Figure Exp.25), to demonstrate the possible amplitude of the adjustments realized by DECOMPXR.  Maximum precision is reached after 10 iterations with the following values:

Position	Intensity	FWHM	Shape K1	Shape K2

8.817	8889	0.302	Gaussian	Gaussian

8.577	10975	0.791	Gaussian	Gaussian

8.157	7438	1.584	Gaussian	Gaussian

The resulting fit is good (Figure Exp.26); then, you just have to add back the 6.25 °2 peak to get a complete set of initial values on the complete angular range (Figure Exp. 27).

Position	Intensity	FWHM	Shape K1	Shape K2

8.817	8889	0.302	Gaussian	Gaussian

8.577	10975	0.791	Gaussian	Gaussian

8.157	7438	1.584	Gaussian	Gaussian

6.25	2200	0.65	Gaussian	Lorentzian

After 14 additional iterations, maximum precision is reached with the same values than the other procedure (Figure Exp.28 and Figure Exp.23):

Position	Intensity	FWHM	Shape K1	Shape K2

8.822	8408	0.288	Gaussian	Gaussian

8.608	9762	0.718	Gaussian	Gaussian

8.245	9091	1.447	Gaussian	Gaussian

6.385	2224	1.027	Gaussian	Lorentzian

Conclusion: These examples are meant to help you become familiar with the logic of DECOMPXR, and of the decomposition treatment.  They provide you with a few tips resulting of five years of intensive use to make DECOMPXR an easy-to-use tool.  For example, in case of divergence problems, it may be useful to perform a "manual adjustment" of the initial values set, or to run a few hundreds iterations with the non linear simplex method to start with initial values closer to the optimal values.  When the calculated values give a correct fit but maximum precision can not be reached, it may be interesting to use a larger angular range, or to print intermediate results (Parameters+Output+Total): calculated values may oscillate between two sets of values, usually very similar.  By resetting initial values in between the two sets, it is usually possible to reach maximum precision after a few tries.

However, there are no absolute rules as far as methodology, determination of initial values, or peak shape go.  Consequently, this program is very interactive and simple on purpose to help you build your own trial-and-error experience and thus your own judgement and your own methodology specifically adapted to your material.

We hope you will enjoy this program as much as we do.  If you have any comments or suggestions please contact:

Dr. Alain Bouchet	Dr. Bruno Lanson
E.R.M. - Batiment GON	Environmental Geochemistry Group
Faculté des Sciences	LGIT - IRIGM

40 Avenue Recteur Pineau	BP 53
86022 Poitiers cedex	38041 Grenoble cedex 9
FRANCE	FRANCE

Phone# +33 (0)5 49 46 18 11	Phone# +33 (0)4 76 82 80 17

Fax# +33 (0)5 49 45 40 26	Fax# +33 (0)4 76 82 81 01

E-mail : Bruno.Lanson@obs.ujf-grenoble.fr

REFERENCES

Beaufort D., Baronnet A., Lanson B., & Meunier A. : Corrensite: A single phase or a mixed-layered phyllosilicate of the saponite-chlorite conversion series? The case study of the Sancerre-Couy deep drill-hole (France). American Mineralogist, 82,110-125.

Bouchet, A., Lajudie, A., Rassineux, F., Meunier, A., and Atabek, R. (1992) Mineralogy and kinetics of alteration of a mixed-layer kaolinite/smectite in nuclear waste disposal simulation experiment (Stripa site, Sweden): Applied Clay Science, 7, 113-123.

Freed, R.L., and Peacor, D.R. (1992) Diagenesis and the formation of illite-rich I/S crystals in Gulf Coast shales: TEM study of clay separates: Journal of Sedimentary Petrology, 62, 220-234.

Howard, S.A., and Preston, K.D. (1989) Profile fitting of powder diffraction patterns: in Modern powder diffraction, Bish, D.L., and Post, J.E. eds., Mineralogical Society of America, 20, 217-275.

Howard, S.A., and Snyder, R.L. (1983) An evaluation of some profile models and optimization procedures used in the profile fitting: Advances in X-ray analysis, 26, 73-81.

Jones RC. 1989. A computer technique for X-ray diffraction curve fitting/peak decomposition. In: Pevear DR, Mumpton FA, editors. Clay Minerals Society workshop lectures, vol 1: Quantitative mineral analysis of clays. Boulder, Co: Clay Miner Soc. p 51-101.

Lanson, B. (1990) Mise en évidence des mécanismes réactionnels des interstratifiés illite/smectite au cours de la diagenèse: Ph. D. thesis, University of Paris, France, 366 pp.

Lanson, B. (1992) Application de la décomposition des diffractogrammes de rayons-X à l'identification des minéraux argileux: Comptes-rendus du Colloque rayons-X Siemens, Paris, vol. 2 - Diffraction-X, Siemens ed., 62-72.

Lanson, B. (1997) Decomposition of experimental X-ray diffraction profile (profile fitting): A convenient way to study clay minerals: Clays & Clay Miner., in press.

Lanson, B., Beaufort, D., Berger, G., Baradat, J., and Lacharpagne, J.C. (1996) Late-stage diagenesis of clay minerals in porous rocks: Lower Permian Rotliegendes reservoir off-shore of The Netherlands : J. Sed. Res., 66, 501-518.

Lanson, B., and Besson, G. (1992) Characterization of the end of the smectite-to-illite transformation: Decomposition of X-ray patterns: Clays & Clay Minerals, 40, 40-52.

Lanson, B., and Champion, D. (1991) The I/S-to-illite reaction in the late stage diagenesis: American Journal of Science, 291, 473-506.

Lanson, B., and Velde, B. (1992) Decomposition of X-ray diffraction patterns: A convenient way to describe complex I/S diagenetic evolution: Clays & Clay Minerals, in press.

Matthews J, Velde B, Johansen H. 1994. Significance of K-Ar ages of authigenic illitic clay minerals in sandstones and shales from the North Sea. Clay Miner 29:379-389.

Righi, D., and Meunier, A. (1991) Characterization and genetic interpretation of clays in an acid brown soil (dystrochrept) developed in a granitic saprolite: Clays & Clay Minerals, 39, 519-530.

Righi D, Petit S, Bouchet A. 1993. Characterization of hydroxy-interlayered vermiculite and illite/smectite interstratified minerals from the weathering of chlorite in a cryothod. Clays Clay Miner 41:484-495.

Righi D, Velde B, Meunier A. 1995. Clay stability in clay-dominated soil systems. Clay Miner 30:45-54.

Robinson D, Bevins RE. 1994. Mafic phyllosilicates in low-grade metabasites. Characterization using deconvolution analysis. Clay Miner 29:223-237.

Stern, W.B., Mullis, J., Rahn, M., and Frey, M. (1991) Deconvolution of the first "illite" basal reflection: Schweiz. Miner. Petrogr. Mitt., 71, 453-462.

Varajao A, Meunier A. 1995. Particle morphological evolution during the conversion of I/S to illite in lower cretaceous shales from Sergipe-Alagoas Basin, Brazil. Clays Clay Miner 43:14-28.

Velde, B. (1985) Clays minerals: A physico-chemical explanation of their occurrence: in Development in Sedimentology, 40, Elsevier ed., 427 pp.

ANNEX 1 - FILE FORMATS

Format of the configuration file DECOMP.CFG:

CONFIGURATION FILE FOR X-RAY PATTERNS DECOMPOSITION PROGRAM
DECOMPXR - JULY 1992 version

N.B. This file must be in the directory from which DECOMP is run

1) Protection code - Check the installation procedure

12345678

2) Default radiation (Cu, Co, Fe, Cr, Mo) :

Cu

3) Output level (3 : None ; 2 : Minimum ; 1 : Total) :

2

4) Path for temporary files - A virtual drive is recommended:

F:\

5) Monochromatic radiation (0:No; 1: Yes)

0

Format of a data file:

601   6.0000    0.0200    4488

0

350 M CHV AD RAIE 001 / 03-MAY-91

2.0000

0.000

0.000

0.000

45131. 43845. 42983. 41785. 40855.

40119. 39304. 38328. 38062. 37249.

(.....)

3524.  3447.  3403.  3285.  3305.

3402.  3368.  3220.  3192.  3272.

3066.

601: Number of  data points defining the pattern.  Absolute maximum is 2801.

6.0000: Counting time per step (must be present but the value is ignored)

0.0200: Step size in °2.

4488: Internal value, must be present but the exact value is ignored.

0: Internal value, must be present but the exact value is ignored.

350 M CHV AD RAIE 001 / 03-MAY-91: Sample name.

2.0000: Lower limit of the pattern in °2.

0.000: Internal value, must be present but the exact value is ignored.

0.000: Internal value, must be present but the exact value is ignored.

0.000: Internal value, must be present but the exact value is ignored.

Then all intensities measured are given, with a separator (blank, comma, ...) between two consecutive values.  The number of measured intensities per line is not strictly formatted.

Format of a result file:

C:\DECOMP\CHV16451.DAT

4.50000000000000E+0000

1.05000000000000E+0001

4

6.38511875893710E+0000

2.22446650536642E+0003

1.02680862298392E+0000

G

L

0

(.....)

2.88044562151510E-0001

G

G

1

C:\DECOMP\CHV16451.DAT: Name of the data file to decompose.

4.50000000000000E+0000: Lower limit of the angular range to decompose.

1.05000000000000E+0001: Upper limit of the angular range to decompose.

4: Number of elementary curve to fit on the experimental pattern.

6.38511875893710E+0000: Position of the K1 contribution of the first elementary curve to fit on the experimental pattern.  Value in °2.

2.22446650536642E+0003: Intensity of the K1 contribution of the first elementary curve to fit on the experimental pattern.  Value in total counts.

1.02680862298392E+0000: Width of the K1 contribution of the first elementary curve to fit on the experimental pattern.  Value in °2.

G: Shape of the K1 contribution of the first elementary curve to fit on the experimental pattern.  G for gaussian.

L: Shape of the K2 contribution of the first elementary curve to fit on the experimental pattern.  L for lorentzian.

0: Fixed peak parameter; 0 fitted peak - 1 fixed peak.

Format of the command files used by DECOMPXR:

oBHV16451.DC4oBHV350A1.DC4

oBHV16451.DC4: Name of a data file if the command file is to be used for automatic background stripping; name of a result file if the command file is to be used for automatic calculation.  Both types or command files are created with LIST (DECOMP option).

Format of the command files used by translation programs:

2

BHV16451

BHV350A1

2: Number of data files to translate.

BHV16451: Name of the data files to transfer without any extension, the default extensions being added by the appropriate translator program.  These command files may be created with LIST (TRANSCOD option).

INDEX

Angular range:	17, 21

Automatic calculation:	20, 25-26

Background stripping:	19, 20, 21-22, 25

Batch calculation:	20, 25-26

Command files:

creation:	31

format:	46

Configuration:

procedure:	13-14

file DECOMP.CFG:	44

Current directory:	17

Data files:

edit:	15

format:	44-45

load:	16

DECOMP.CFG:	13, 14, 15, 16, 18-19, 44

Decomposition:

examples:	5-6, 34-40

procedure:	20-25, 34-40

references:	5-6, 42-43

Divergence:	24

Error messages:	27-31

Entry edition:	16

File:

list:	9-10

formats:	44-46

Format:

command files:	46

configuration file:	44

data files:	44

result files:	45

translation:	32-33

Hardcopy:	8, 24, 27

Initial values:	22-23, 34-40

Installation procedure	11-14

K2 contribution:	18, 23, 34

Non linear simplex:	24-25

Number:

elementary curves:	18, 35

iterations:	18

data points in data file	16, 29

data points in range	17, 22, 28

Output level:	18

Peak shape:	35

Pre treatment: 	19, 21-22, 25-26

Preview:	23

Protection	14

Requirements:	7

Radiation:

monochromatism:	18, 19

type:	19

Result files:

creation:	20, 23

format:	45

how to use:	22, 25

Simulation:	19

Singular matrix:	24

Smoothing:	19, 21, 25

Temporary files:	7-8

Virtual drive:	7-8

Addition to the User’s guide

1) The surface area and the relative area for each elementary peaks are printed in the result section if printing is enabled (see Parameters Output - User’s guide p. 18). The surface area is determined by numerical integration over the fitted angular range and is independent of the step size used during data collection.

2) The <F6> key to set fixed the parameters of an elementary peak is not just a On/Off switch any more (see Go - User’s guide p. 23). When you press the <F6> key for the first time you set fixed the complete set of parameters for the concerned peak. A second press set the FWHM and the intensity parameters free but keep the position fixed, while a third press set free again the complete set of parameters. The color of the fixed peak characteristics is switched from yellow to magenta. When printing is enabled, a “FIXED” label is printed for each peak parameter which has not been adjusted during the calculation.

3) The <F7> key is now taken into account and permit in the data entry table to remove the current elementary peak whatever its position in the table. Please note that this key is disabled when only one elementary peak is present in the table.

4) A new reliability factor has been put up recently , and is calculated as follows:

RF=k(calc(k)−mean)2k(calc(k)−mean)2+k(calc(k)−exper(k))2

Please note that the higher the RF (ideal = 100%), the better the fit. This reliability factor is printed in the results section as previously. Additionally this RF is displayed in the data entry table to make it easier to follow the improvements induced by preliminary hand made adjustments on initial values.

Microsoft is a registered trademark, MS-DOS, and Windows are trademarks of Microsoft Corporation.

Intel is a registered trademark of Intel Corporation.

IBM is a registered trademark of International Business Machine Corporation.